QMSR (ISO 13485) / 21 CFR 820 Crosswalk

Foundational QMS Documentation — PulseTrack Case Study

NOTE: PulseTrack is a fictional device used for regulatory affairs training purposes. This document is a portfolio exercise, not a real regulatory filing.

# 1. Why This Crosswalk Matters

Effective February 2, 2026, FDA's Quality System Regulation (QSR) was restructured into the Quality Management System Regulation (QMSR), which incorporates ISO 13485:2016 by reference rather than restating equivalent requirements in separate US-specific language. For most clauses, 21 CFR 820 now simply points to the corresponding ISO 13485 clause instead of duplicating the text. A small number of provisions — such as the Device History Record and Device Master Record — were retained as distinct US-specific requirements because FDA determined ISO 13485 did not fully cover them.

Practically, this means a company already certified to ISO 13485 is most of the way to QMSR compliance, but still needs to confirm coverage of the handful of US-specific additions and update internal procedures/inspection-readiness documentation to reference the new structure.

# 2. Crosswalk Table (Illustrative Excerpt)

| QSR Subpart | Legacy 820 Provision | Corresponding QMSR / ISO 13485 Reference | Notes |
| --- | --- | --- | --- |
| Subpart B | 820.20 Management responsibility | Clause 5 — Management Responsibility | QMSR references ISO 13485 Clause 5 directly; FDA retains a few US-specific clarifications (e.g., management review inputs) |
| Subpart B | 820.22 Quality audit | Clause 8.2.4 — Internal Audit | Harmonized; audit program requirements now point to ISO 13485 language |
| Subpart C | 820.30 Design controls | Clause 7.3 — Design and Development | Core design control chain (inputs, outputs, verification, validation, transfer, changes) maps directly to 7.3.2–7.3.10 |
| Subpart E | 820.50 Purchasing controls | Clause 7.4 — Purchasing | Supplier evaluation/re-evaluation requirements now align with ISO 13485 supplier control language |
| Subpart G | 820.70 Production and process controls | Clause 7.5 — Production and Service Provision | Process validation and environmental control requirements harmonized |
| Subpart J | 820.100 Corrective and preventive action (CAPA) | Clause 8.5.2 / 8.5.3 — Corrective/Preventive Action | CAPA process expectations retained; FDA kept its own emphasis on statistical methodology in select areas |
| Subpart J | 820.198 Complaint files | Clause 8.2.2 — Feedback / Complaint Handling | Complaint handling now explicitly tied to the broader feedback loop described in ISO 13485 |
| Subpart M | 820.184 Device history record (DHR) | Retained as US-specific QMSR requirement | One of the areas FDA determined ISO 13485 did not fully cover — kept as an added US-specific clause |
| Subpart M | 820.181 Device master record (DMR) | Retained as US-specific QMSR requirement | Same rationale — DMR concept preserved outside pure ISO 13485 text |

# 3. Practical Impact for PulseTrack's QMS

For Vitalis Health's quality system, this transition means: (1) internal SOPs are being reviewed to reference ISO 13485 clause numbers alongside legacy 820 citations during the transition period; (2) the Device History Record and Device Master Record are maintained as distinct controlled documents, since these were retained as US-specific QMSR requirements; and (3) internal audit checklists were updated to reflect FDA's revised, risk-based inspection process rather than the legacy Quality System Inspection Technique (QSIT).